# PHAN TICH MODULE CAMERALANE (camera_lane.hpp / camera_lane.cpp)

Tai lieu mo ta phuong phap phat hien va bo tim duong lane cua module CameraLane. Noi dung dong bo voi source hien tai: nhanh chinh giu nguyen pipeline cua master opencv/AdvancedLaneDetection, chi doi nguong mau cho 2 vach cao su den tren san gach kem.

# MUC LUC

1. Tong quan: module CameraLane lam gi

2. Vi tri trong kien truc he thong va luong du lieu

3. File 1 - camera_lane.hpp: hang so, hop dong dau ra, bien thanh vien

4. File 2 - camera_lane.cpp: luong xu ly va cac nhom ham

5. Phuong phap phat hien lane (nhanh chinh IPM cua master)

6. He toa do: anh goc - bird's-eye - duong cong duoc chieu nguoc

7. Cac lop chong nhieu va dieu kien tu choi

8. May trang thai ba muc do toc do

9. Tu do lech do duoc den lenh lai gui xuong ESP32

10. Bang tham so day du (trich tu camera_lane.hpp)

11. Van de da phat hien, gioi han va huong sua

12. Ket luan va bang kiem chung

# 1. Tong quan: module CameraLane lam gi

## 1.1. Ban chat thuat toan: hai nhanh, nhanh chinh theo master

Module nay tim duong lane bang cong vien cua camera va gui diem lech + lenh lai xuong ESP32 qua UART. Thuật toán gồm hai nhánh, nhánh nào cũng chốt an toàn bằng quy tắc 'khong co du lieu thi khong ra lenh lai':

| Nhanh | Vai tro | Khi nao dung |
| --- | --- | --- |
| A - IPM (chinh) | Pipeline master: Sobel X AND nguong mau -> che nap xe -> IPM bird's-eye -> 9 cua so truot -> fit bac hai tren pixel -> chieu nguoc ve anh goc | Mac dinh khi hai vach duoc nhin thay va viet trong bird view |
| B - CANNY + quet dong | Du phong: Canny tren anh xam ROI, quet tung dong tim bien trai/phai, loi toi o 5 diem gan xe | Khi nhanh A that bai (lanh lech tam, vach qua hep, mau sac bat thuong) |

Khac voi master, module bo sung (1) loai blob nhieu, (2) kiem tra do lech cua fit, (3) kiem tra be rong lan de khong bao gio steer bang duong cong sai, (4) nhanh Canny du phong. Cac buoc bo sung nay chi loai bo ket qua khong hop le, khong doi thu tu hay cong thuc cua master.

## 1.2. Quy trinh xu ly 10 buoc

Chot mau dau ra (STOP, dev = 0, speed = 0) truoc moi khi xu ly.

Tinh ROI theo toc do (chi dung cho nhanh du phong B).

Nhanh A: tach kenh BGR, CLAHE tren kenh G, Sobel X, ghep bang bitwise_and voi nguong kenh R (LANE_MIN_VALUE..LANE_MAX_VALUE).

Che nap xe (bonnet), morphology close + open, loai blob theo dien tich/chieu cao/bien Sobel.

IPM: ghep 4 diem nguon cua master thanh anh bird's-eye 320x240; che nap xe lan nua.

Histogram cua nua duoi bird view tim diem xuat phat cua hai cua so.

9 cua so truot margin 50 px theo chieu doc len, moi cua so lay trung tam vach va gom pixel de fit.

Fit bac hai tren TOAN BO pixel lane, kiem tra residual tren trung tam cua so.

Kiem tra be rong lan (bird view va anh goc) de chan hai lan dinh nhau.

Chieu nguoc duong cong ve anh goc, dua ve luoi y chung, tinh dev_final_px, lenh lai va toc do muc tiep.

## 1.3. Vi sao tach hai luong (capture / process)

capture_thread chi doc frame vao bo dem kich thuoc 2, process_thread giai ma va xuat. Doc va giai tach roi nhau de viec giai ma khong lam mat frame va de do tre khong cong don lau vao thoi gian thu thap.

# 2. Vi tri trong kien truc he thong va luong du lieu

Chuoi khoi tao va chuoi tieu thu ket qua moi vong:

camera_node (VideoCapture 640x400 @120fps)
   -> CameraLane::capture_thread  -> FrameSlot[FRAME_BUFFER_COUNT]
   -> CameraLane::process_thread -> detect_lanes() -> LaneOutput
   -> /camera/lane_output (dinh ky)
   -> fusion_viz_node: ve overlay
   -> uart_bridge_node: -> ESP32 (CMD_STEER / SPEED) + lidar_no_vehicle_override

## 2.1. Bang anh xa truong LaneOutput

| Truong | Y nghia | Noi su dung |
| --- | --- | --- |
| left / right | Diem cua hai duong lane tren anh goc (px) | Ve overlay, tinh centerline |
| center | Diem canh giua hai lan | Ve overlay, kiem tra do lech |
| dev_final_px | Lech canh giua so voi chinh giua khung hinh (am = phai sang trai) | Lenh lai ESP32 |
| is_dual_lane | Co tim thay ca hai vach hay khong | Chon nhanh tinh dev, do tin cay |
| pixels_used | Tong so diem lane duoc dung | Chong nhieu, ghi log |
| camera_cmd | STOP / FWD / LEFT / RIGHT | Goi xuong ESP32 |
| target_speed_x10 | Toc do muc tieu x10 (85 = 8.5 km/h) | Bộ lap toc do |
| curve_angle_deg | Goc nghiep voi truc ngang (do) | Bộ lap toc do |
| speed_factor | He so lam cham (0..255) | Bộ phoi toc do voi LiDAR |
| detector_mode | 0 = scanline du phong, 1 = IPM chinh | Che do do kinh |
| processing_ms | Thoi gian xu ly mot khung (ms) | Hien thi |
| lane_color_bgr / lane_color_name | Mau vach do duoc o giua anh | Chon nguong mau linh hoat |

# 3. File 1 - camera_lane.hpp: hang so, hop dong dau ra, bien thanh vien

## 3.1. struct LaneOutput

struct LaneOutput la hop dong dau ra cua module. Mac dinh an toan: valid = false, camera_cmd = "STOP", dev_final_px = 0, target_speed_x10 = 0. Truc khi co du lieu lane hop le, module khong bao gio giu mot lenh lai cu.

## 3.2. Nhom hang so phuc vu phuong phap master

Cac hang so nay quyet dinh toan bo hanh vi cua nhanh IPM:

| Hang so | Gia tri | Y nghia |
| --- | --- | --- |
| FRAME_W | 640 |  |
| FRAME_H | 400 |  |
| SOBEL_MIN_THRESHOLD | 30 |  |
| SOBEL_MAX_THRESHOLD | 255 |  |
| SOBEL_X_PEAK_MIN | 60.0f | Blob khong co bien Sobel du manh bi loai (bong do, vet sang) |
| LANE_MIN_VALUE | 0 | Vach den: nguong duoi kenh R (0) - doi chieu voi 100 cua master |
| LANE_MAX_VALUE | 110 | Vach den: nguong tren kenh R (110) - san gach kem ~225 nen tach duoc |
| CLAHE_CLIP_LIMIT | 1.5f |  |
| CLAHE_GRID_X | 8 |  |
| CLAHE_GRID_Y | 8 |  |
| BONNET_ENABLE | true | Che vung nap xe, giong master |
| BONNET_X0 | 0.3125f |  |
| BONNET_X1 | 0.6250f |  |
| BONNET_Y0 | 0.6481f |  |
| MASK_CLOSE_X | 7 |  |
| MASK_CLOSE_KERNEL | 21 |  |
| MASK_OPEN_KERNEL | 3 |  |
| MIN_BLOB_AREA | 60 |  |
| MIN_BLOB_HEIGHT | 10 |  |
| MAX_BLOB_WIDTH_PX | 260.0f |  |

## 3.3. Nhom hang so cua IPM va cua so truot

| Hang so | Gia tri | Y nghia |
| --- | --- | --- |
| BIRD_W | 320 |  |
| BIRD_H | 240 |  |
| IPM_TL_X | 0.4479f | Canh tren cua hinh thang (trai) - gia tri cua master |
| IPM_TR_X | 0.5521f | Canh tren cua hinh thang (phai) - gia tri cua master |
| IPM_BL_X | 0.1146f | Canh day (trai) - gia tri cua master |
| IPM_BR_X | 0.9271f | Canh day (phai) - gia tri cua master |
| N_WINDOWS | 9 | So cua so truot doc len (master: 9) |
| WINDOW_MARGIN | 50 | Ban kinh tim kiem quanh seed (master: 50) |
| WINDOW_MIN_POINTS | 8 | So diem toi thieu de coi cua so la co du lieu |
| WINDOW_MAX_SHIFT | 30 | Tran dich chuyen toi da cua seed giua cac cua so |
| WINDOW_RUN_RATIO | 0.25f | Nguong tim do rong run quanh dinh cot |
| BIRD_HIST_TOP_FRAC | 0.5f | Histogram tinh tren nua duoi bird view |
| BIRD_MIN_COLUMN_RATIO | 0.15f | Cot phai dat nguong diem moi duoc dung lam seed |
| BIRD_VALLEY_RATIO | 0.60f | Thung thap giua hai dinh histogram phai ton tai |
| BIRD_SIDE_TOLERANCE | 0.06f |  |
| BIRD_MIN_HALF_WIDTH | 20 |  |
| BIRD_XMID_JUMP_MAX | 60 |  |
| BIRD_MIN_LANE_WIDTH | 40.0f |  |
| BIRD_MAX_LANE_WIDTH | 300.0f |  |
| BIRD_MAX_FIT_RESIDUAL_PX | 10.0f | Sai so toi da cho phep giua duong fit va trung tam cua so |
| BIRD_MIN_FIT_POINTS | 5 | So cua so toi thieu moi lan truoc khi fit |
| BIRD_MIN_FIT_PIXELS | 60 | So pixel toi thieu moi lan truoc khi fit |
| IPM_MIN_WIDTH_RATIO | 0.60f | Be rong do duoc trong anh goc so voi uoc luong |
| IPM_MAX_WIDTH_RATIO | 1.45f | Be rong do duoc trong anh goc so voi uoc luong |
| BIRD_SAMPLE_TOP_FRAC | 0.25f |  |
| BIRD_SAMPLE_STEP | 4 |  |
| IMAGE_SAMPLE_STEP | 4 |  |
| BIRD_LANE_BAND_MARGIN | 33 |  |
| BIRD_LANE_BAND_ALPHA | 0.30f |  |
| VIEW_BIRD_EYE | 0 |  |
| VIEW_NORMAL | 1 |  |

Ghi chu: khong con BIRD_FIT_EXTRAP_ROWS hay WINDOW_RUN_WIDTH_RATIO. Master danh gia da thuc tren toan bird view, va do rong run cua mot cua so khong phai dau hieu hai lan dinh nhau (mot cua so chua ca hai bien cua vach) nên da bo hanh muc do rong run.

## 3.4. Nhom hang so cua nhanh du phong CANNY + quet dong

| Hang so | Gia tri | Y nghia |
| --- | --- | --- |
| CANNY_LOW | 30 |  |
| CANNY_HIGH | 120 |  |
| BLUR_KERNEL | 5 |  |
| SCAN_STEP | 4 |  |
| MIN_PRELIM_POINTS | 15 |  |
| MIN_COMMON_POINTS | 10 |  |
| MIN_FINAL_POINTS | 5 |  |
| ROI_FACTOR_SLOW | 0.70f | He so chiet ROI khi chay cham (nhanh du phong) |
| ROI_FACTOR_FAST_MIN | 0.55f | He so chiet ROI khi chay nhanh |
| ROI_SPEED_START | 4.0f |  |
| ROI_SPEED_GAIN | 0.02f |  |
| SINGLE_ROI_FACTOR | 0.85f | Chi lay vung gan xe hon khi chi thay mot vach |
| MIN_LANE_WIDTH_PX | 200.0f |  |
| MAX_LANE_WIDTH_PX | 500.0f |  |
| DEFAULT_LANE_WIDTH_PX | 230.0f |  |
| ALPHA_LANE_WIDTH | 0.20f | He so loc so sanh be rong lan |
| ADAPTIVE_BLOCK | 41 |  |
| ADAPTIVE_C | 8 |  |

## 3.5. Bien thanh vien dang nho

| Bien | Vai tro |
| --- | --- |
| mask_ | Mask lane cua nhanh A (Sobel AND mau) |
| sobel_x_ | Sobel X cua kenh G, dung kiem chung blob va ve giao dien |
| eq_ | Anh sau CLAHE |
| bird_ | Mask trong khung bird's-eye 320x240 |
| gray_ | Anh xam cua dai ROI cho nhanh du phong |
| gray_full_ | Anh xam toan khung cho fallback adaptive cua mask |
| left_fit_ / right_fit_ | He so bac hai [a, b, c] cua duong fit trong bird view |
| left_fit_px_ / right_fit_px_ | Toan bo pixel lane thu duoc trong cac cua so |
| left_win_means_ / right_win_means_ | Trung tam cua tung cua so (don vi px bird view) |
| left_raw_ / right_raw_ | Duong cong da chieu nguoc ve anh goc, dung de lam trun vị |
| lane_width_est_px_ | Be rong lan uoc luong, dung cho nhanh mot vach va kiem tra IPM |
| xmid_scan_ | Vi tri quet gan nhat cua nhanh du phong |
| speed_state_ | Trang thai toc do hien tai |

# 4. File 2 - camera_lane.cpp: luong xu ly va cac nhom ham

| Nhom ham | Vai tro |
| --- | --- |
| capture_thread / process_thread | Doc frame vao bo dem; giai ma va goi detect_lanes |
| detect_lanes | Kiem tra dau vao, chon nhanh A hoac B, tinh dev / lenh / toc do, tao anh visualize |
| sobel_color_thresholding | Sobel X tren kenh G + nguong kenh R, ghep bitwise_and (dung ham cua master) |
| mask_bonnet | Xoa vung nap xe trong mask |
| filter_lane_blobs | Morphology + loai blob theo dien tich, chieu cao, do rong, bien Sobel; fallback adaptive threshold |
| update_ipm | Tinh ma bien doi thang tu 4 diem nguon cua master va ma chieu nguoc |
| transforming_view | Warp mask sang bird's-eye hoac nguoc lai ve anh goc |
| window_search / search_window | Histogram + 9 cua so truot, cap nhat seed va gom pixel |
| fit_poly / fit_residual_ok | Fit bac hai va kiem tra sai so tren trung tam cua so |
| build_curve_points | Danh gia da thuc tren bird view roi chieu nguoc ve anh goc |
| resample_curve / median_smooth / median_tail_x | Dua hai lan ve cung luoi y, lam trun vị |
| detect_lanes_scanline | Nhanh du phong Canny + quet dong theo ROI |
| draw_lane_band | To bang lane theo cua so cua master de kiem soat bang mat |
| calculate_slope / calculate_slope_offset | Goc duong cong va bu som do lech xa |
| calculate_target_speed | May trang thai chon toc do muc tieu |

Thu tu goi ham trong detect_lanes (dong thu tu quan trong):

sobel_color_thresholding(bgr)   // master: sobel AND color, tren TOAN khung
filter_lane_blobs(gray_full_)  // morphology + loai nhieu
detect_lanes_ipm(bgr, ...)     // master: IPM -> window search -> fit
  -> neu that bai: detect_lanes_scanline(roi, ...)  // du phong
median_smooth / dev_final_px / camera_cmd / calculate_target_speed

# 5. Phuong phap phat hien lane (nhanh chinh IPM cua master)

## 5.1. Buoc 1 - tao mask bang AND hai nguong (master)

clahe_->apply(G, eq_);
Sobel(eq_, sobel_x_, CV_16S, 1, 0, 3);
convertScaleAbs(sobel_x_, sobel_x_);
inRange(sobel_x_, SOBEL_MIN_THRESHOLD, SOBEL_MAX_THRESHOLD, sobel_mask);
inRange(R, LANE_MIN_VALUE, LANE_MAX_VALUE, color_mask);
bitwise_and(sobel_mask, color_mask, mask_);   // sobelColorThresholding cua master

Master dung colorMin = 100, colorMax = 255 cho lan trang tren duong toi. O day hai vach cao su DEN tren san gach kem nen doi chieu: LANE_MIN_VALUE = 0, LANE_MAX_VALUE = 110. Cac buoc con lai giu nguyen: cung kernel Sobel X 3x3, cung nguong Sobel 30..255, cung kenh G lam kenh bien. CLAHE (1.5f, lưới 8x8) chi them để bù đèn trần phòng lab không đều.

## 5.2. Buoc 2 - che nap xe (bonnet)

Master cho mat bonnet (300,350)-(600,540) tren anh 960x540. Quy veo ty le: x0 = 0.3125f, x1 = 0.6250f, y0 = 0.6481f. Vung nay la nua giua phia duoi khung; duong cong fit trong vung nay la ngoai suy da thuc, nen phai che de tranh tinh sai o chay thu.

## 5.3. Buoc 3 - IPM: tu anh goc sang bird's-eye

IPM dung 4 diem nguon cua master, tinh theo ty le so voi khung hinh (nen khong phu thuoc do phan giai):

| Diem | Ty le X | Ty le Y |
| --- | --- | --- |
| Canh tren - trai (IPM_TL_X) | 0.4479f | 0.4479f |
| Canh tren - phai (IPM_TR_X) | 0.5521f | 0.4479f |
| Canh day - trai (IPM_BL_X) | 0.1146f | 0.9271f |
| Canh day - phai (IPM_BR_X) | 0.9271f | 0.9271f |

Gia tri goc cua master: (110,540) (430,330) (530,330) (890,540) tren anh 960x540. Bird view kich thuoc 320x240. Ma bien doi va ma chieu nguoc duoc tinh lai khi kich thuoc khung doi va giu trong update_ipm().

## 5.4. Buoc 4 - tim diem xuat phat bang histogram (master)

Tinh tong so pixel theo cot tren nua duoi bird view (BIRD_HIST_TOP_FRAC = 0.5f). Diem xuat phat cua hai cua so la cot lon nhat ben trai va ben phai, sau khi kiem tra: cot phai du nguong (BIRD_MIN_COLUMN_RATIO), phai co mot thung thap giua hai dinh (BIRD_VALLEY_RATIO), va hai dinh phai tach nhau it nhat BIRD_MIN_HALF_WIDTH px.

## 5.5. Buoc 5 - 9 cua so truot doc len

Chia bird view thanh 9 cua so, di tu duoi len. Moi cua so quet trong ban kinh 50 px quanh seed, tim cot co tong diem lon nhat, noi thanh run (nguong 0.25f lan dinh), lay trung tam run la vi tri moi. Cua so bi bo qua neu: so diem it hon 8, hoac trung tam chuyen qua xa seed tren 30 px.

Mot cua so co du lieu se gom TOAN BO pixel lane nam trong run vao tap diem fit. Day la diem khac biet so voi master (master chi giu 9 trung tam): gom pixel lam fit benh hon nhieu va it nhieu khi vach ngoai chuong.

## 5.6. Buoc 6 - fit bac hai va kiem tra do chap

Fit x = a + b*y + c*y^2 tren tap pixel cua moi lan (y chuan hoa theo BIRD_H). Lan chi duoc dung khi: so pixel >= 60 va so cua so co du lieu >= 5. Do lech lon nhat giua duong fit va trung tam cua so phai khong vuot 10.0f px, neu vuot thi bo fit (nghi doan vach bi dinh, nhiu rieng lam lung lay cua so so).

## 5.7. Buoc 7 - kiem tra be rong lan

Bề rộng đo trong bird view phải nằm trong [40.0f, 300.0f]. Bề rộng cùng đo đó được chiếu ngược xuống ảnh gốc rồi so với ước lượng đang lọc theo tỉ lệ [0.60f, 1.45f] - vì số đo luôn nhỏ hơn bề rộng thật do chỉ lấy trong vùng cửa sổ. Qua hai kiểm tra này, cap xmid va be rong uoc luong moi duoc cap nhat, va ket qua moi duoc gan nhan is_dual_lane = true.

## 5.8. Buoc 8 - chieu nguoc duong cong ve anh goc

Da thuc duoc danh gia tren toan bo bird view (nhu master, khong cat va khong noi tuyet tinh o vung ngoai du lieu), lay mau moi 4 px, chieu nguoc bang ma perspectiveTransform roi dua ve anh goc. Nhanh IPM bao cao TOAN BO duong cong (tu dong tren khung den dong cuoi), khong cat theo ROI: vay con ngay duoi lai suy tu vung du lieu that.

Hai lan duoc dua ve cung luoi y (IMAGE_SAMPLE_STEP = 4 px) de tinh canh giua. neu mot lan khong du diem thi suy trung tam tu vi tri canh vach va be rong uoc luong. dev_final_px lay trung vij cua 5 diem gan xe.

## 5.9. Buoc 9 - nhanh du phong khi IPM that bai

Neu mask rong, khong con cua so nao co du lieu, fit qua nghia hoac be rong lan vo ly, module chuyen sang nhanh CANNY + quet dong tren dai ROI (ROI_FACTOR_* theo toc do). Nhanh nay dung Canny (30..120) tren anh xam da lam mo, quet tung dong tu xmid_scan_ ra hai ben, yeu cau be rong trong [200.0f, 500.0f] de coi la lan kep, cap nhat xmid theo gia tri do duoc, roi lam trun vị 3 diem va cat theo y0_used.

# 6. He toa do: anh goc - bird's-eye - duong cong duoc chieu nguoc

| Khung toa do | Dung cho |
| --- | --- |
| Anh goc 640x400 | Hien thi, ve bang lane, tinh dev_final_px |
| Bird's-eye 320x240 | Tim vach, fit bac hai, do be rong lan |
| Luoi y chung cua hai lan | Tinh canh giua va do lech bang trung vij |

Mo yeu toa do chinh la goc tien hoa cua camera (van toc): o bird view hai bien song phai thang va song song. Do lech doc giua cac diem do duoc o cac dong lien tiep giup do la bang chung IPM hieu dung. Do lech lon ngang (lanh gom lai) chinh la truong hop kiem tra be rong lan.

# 7. Cac lop chong nhieu va dieu kien tu choi

| Lop | Cau hinh | Vi du nhi canh |
| --- | --- | --- |
| CLAHE tren kenh G | 1.5f / luoi 8x8 | Vung den tran lam vach mat bien |
| Nguong mau kenh R | 0..110 | Mau san kem ~225 khong lot vao mask |
| AND voi Sobel X | nguong 30 | Vet toi khong co bien manh bi loai |
| Morphology | close 21x7, open 3x3 | Magh m vach, loai dot nho |
| Loai blob | dien tich >= 60, chieu cao >= 10, be rong <= 260.0f | Duong ch e, vet sang |
| Fallback adaptive | block 41, C 8 | Dang sáng lạ khong bat duoc bang dai co dinh |
| Trung vị 3 diem | loc nhiem don le | Mot diem lech |
| Residual cua fit | <= 10.0f px | Vach dinh / nhieu lam lung lay cua so |
| Be rong lan | [40.0f, 300.0f] va ti le [0.60f, 1.45f] | Hai lan dinh nhau, hoac fit sai |
| Trung vij 5 diem gan xe | lay do lech de ra lenh | Diem sai o duoi cua khung |
| Nguong so diem toi thieu | 5 diem / 5 cua so | Khong du du lieu de lenh |

# 8. May trang thai ba muc do toc do

| Trang thai | Dieu kien vao | Dieu kien ra | Toc do |
| --- | --- | --- | --- |
| STRAIGHT | do lech < 40 px va goc < 0.55f | lech >= 50 px hoac goc >= 0.65f | 85 (8.5 km/h) |
| CURVE | lech >= 50 px hoac goc >= 0.65f | gia > 40 va goc < 0.55f | 60 (6.0 km/h) |
| SHARP | gia > 80 va goc >= 0.85f | gia < 65 hoac goc < 0.75f | 45 (4.5 km/h) |

Hysteresis hai bien (lech va goc) tranh cho xe dao chuyen trang thai lien tuc o ranh gioi. Toc do muc tieu duoc lam mem bang ALPHA_TARGET_SPEED = 0.70f truoc khi gui.

# 9. Tu do lech do duoc den lenh lai gui xuong ESP32

| dev_final_px | camera_cmd | Y nghia |
| --- | --- | --- |
| ¦dev¦ < 15 | FWD | Di thang |
| dev < -15 | LEFT | Canh giua lach sang trai (am) |
| dev > 15 | RIGHT | Canh giua lach sang phai |
| valid = false | STOP | Khong du du lieu - khong duoc phanh theo lenh nhung van STOP |

dev_final_px tinh tu trung vij 5 diem gan xe cua duong cong (hoac canh giua hai lan). O nhanh mot vach, trung tam duoc suy tu vi tri vach + nua be rong uoc luong; nhanh nay khong cong bu duong cong vi uoc luong be rong da lam muot phan tuyet doi phan cong.

Chuoi day du: CameraLane -> topic /camera/lane_output -> uart_bridge_node -> khung UART [START][CMD][LEN][DATA][CRC][END] -> ESP32 (nhac dem van toc). Chuong trinh goc tai liu (khong co vat can) co the ghi de lenh bang co che lidar_no_vehicle_override.

# 10. Bang tham so day du (trich tu camera_lane.hpp)

Bang duoi day duoc sinh tu chinh file camera_lane.hpp tai thoi diem tai lieu duoc viet, nen khong the lech voi code.

## 10.1. Toan bo hang so

| Ten hang so | Gia tri |
| --- | --- |
| FRAME_W | 640 |
| FRAME_H | 400 |
| FRAME_BUFFER_COUNT | 2 |
| ENABLE_VISUALIZATION | true |
| ENABLE_DEBUG_LOG | false |
| DEBUG_LOG_EVERY | 15 |
| ENABLE_BIRD_DEBUG | false |
| ENABLE_LANE_COLOR_SAMPLE | true |
| ROI_FACTOR_SLOW | 0.70f |
| ROI_FACTOR_FAST_MIN | 0.55f |
| ROI_SPEED_START | 4.0f |
| ROI_SPEED_GAIN | 0.02f |
| SINGLE_ROI_FACTOR | 0.85f |
| CANNY_LOW | 30 |
| CANNY_HIGH | 120 |
| BLUR_KERNEL | 5 |
| SCAN_STEP | 4 |
| MIN_PRELIM_POINTS | 15 |
| MIN_COMMON_POINTS | 10 |
| MIN_FINAL_POINTS | 5 |
| MIN_LANE_WIDTH_PX | 200.0f |
| MAX_LANE_WIDTH_PX | 500.0f |
| DEFAULT_LANE_WIDTH_PX | 230.0f |
| ALPHA_LANE_WIDTH | 0.20f |
| SOBEL_MIN_THRESHOLD | 30 |
| SOBEL_MAX_THRESHOLD | 255 |
| SOBEL_X_PEAK_MIN | 60.0f |
| LANE_MIN_VALUE | 0 |
| LANE_MAX_VALUE | 110 |
| CLAHE_CLIP_LIMIT | 1.5f |
| CLAHE_GRID_X | 8 |
| CLAHE_GRID_Y | 8 |
| BONNET_ENABLE | true |
| BONNET_X0 | 0.3125f |
| BONNET_X1 | 0.6250f |
| BONNET_Y0 | 0.6481f |
| ADAPTIVE_BLOCK | 41 |
| ADAPTIVE_C | 8 |
| MASK_CLOSE_X | 7 |
| MASK_CLOSE_KERNEL | 21 |
| MASK_OPEN_KERNEL | 3 |
| MIN_BLOB_AREA | 60 |
| MIN_BLOB_HEIGHT | 10 |
| MAX_BLOB_WIDTH_PX | 260.0f |
| BIRD_W | 320 |
| BIRD_H | 240 |
| IPM_TL_X | 0.4479f |
| IPM_TR_X | 0.5521f |
| IPM_BL_X | 0.1146f |
| IPM_BR_X | 0.9271f |
| N_WINDOWS | 9 |
| WINDOW_MARGIN | 50 |
| WINDOW_MIN_POINTS | 8 |
| WINDOW_MAX_SHIFT | 30 |
| WINDOW_RUN_RATIO | 0.25f |
| BIRD_HIST_TOP_FRAC | 0.5f |
| BIRD_MIN_COLUMN_RATIO | 0.15f |
| BIRD_VALLEY_RATIO | 0.60f |
| BIRD_SIDE_TOLERANCE | 0.06f |
| BIRD_MIN_HALF_WIDTH | 20 |
| BIRD_XMID_JUMP_MAX | 60 |
| BIRD_MIN_LANE_WIDTH | 40.0f |
| BIRD_MAX_LANE_WIDTH | 300.0f |
| BIRD_MAX_FIT_RESIDUAL_PX | 10.0f |
| BIRD_MIN_FIT_POINTS | 5 |
| BIRD_MIN_FIT_PIXELS | 60 |
| IPM_MIN_WIDTH_RATIO | 0.60f |
| IPM_MAX_WIDTH_RATIO | 1.45f |
| BIRD_SAMPLE_TOP_FRAC | 0.25f |
| BIRD_SAMPLE_STEP | 4 |
| VIEW_BIRD_EYE | 0 |
| VIEW_NORMAL | 1 |
| BIRD_LANE_BAND_MARGIN | 33 |
| BIRD_LANE_BAND_ALPHA | 0.30f |
| IMAGE_SAMPLE_STEP | 4 |
| FWD_THRESHOLD_PX | 15 |
| SLOPE_OFFSET_START | 0.80f |
| SLOPE_OFFSET_END | 1.00f |
| SLOPE_OFFSET_MIN_PX | 50.0f |
| SLOPE_OFFSET_MAX_PX | 120.0f |
| SPEED_STRAIGHT_X10 | 85 |
| SPEED_CURVE_X10 | 60 |
| SPEED_SHARP_X10 | 45 |
| ALPHA_TARGET_SPEED | 0.70f |
| CURVE_ENTER_SLOPE | 0.65f |
| CURVE_EXIT_SLOPE | 0.55f |
| SHARP_ENTER_SLOPE | 0.85f |
| SHARP_EXIT_SLOPE | 0.75f |
| CURVE_ENTER_DEV_PX | 50 |
| CURVE_EXIT_DEV_PX | 40 |
| SHARP_ENTER_DEV_PX | 80 |
| SHARP_EXIT_DEV_PX | 65 |
| MEDIAN_TAIL_POINTS | 5 |
| MEDIAN_TAIL_POINTS_MAX | 16 |

## 10.2. Ham thanh vien cua class CameraLane

| Ham | Co trong camera_lane.cpp |
| --- | --- |
| start | co |
| stop | co |
| get_latest | co |
| buffered_frames | co |
| capture_thread | co |
| process_thread | co |
| clamp_float | co |
| clamp_int16 | co |
| median_smooth | co |
| median_tail_x | co |
| median_value | co |
| calculate_slope | co |
| calculate_slope_offset | co |
| calculate_target_speed | co |
| poly_eval | co |
| fit_poly | co |
| fit_residual_ok | co |
| sobel_color_thresholding | co |
| mask_bonnet | co |
| filter_lane_blobs | co |
| update_ipm | co |
| transforming_view | co |
| window_search | co |
| search_window | co |
| build_curve_points | co |
| resample_curve | co |
| detect_lanes_ipm | co |
| draw_lane_band | co |
| detect_lanes_scanline | co |
| detect_lanes | co |

# 11. Van de da phat hien, gioi han va huong sua

## 11.1. Hinh hoc IPM phai hieu chinh mot lan khi co frame that

Tu so master khoi tao cho cam bien hanh phuc viem hon: 4 diem nguon IPM va vung che nap xe duoc do tren xe cua master. Camera cua xe ta khac lens, cao goc, do phan giai, nen phai hieu chinh 0.1146f..0.9271f va 0.3125f..0.6481f tren mot frame that truoc khi chay duong. Dac biet: canh tren cua hinh thang cua master chi rong 10.4% khung, o vung xa doanh du kiem rat nhay: lane lech khoi chinh giua anh vai chuc pixel la du de bien mat vach o bird view, khi do module chuyen sang nhanh du phong - an toan nhung lai thong.

## 11.2. Vung duoi nap xe la ngoai suy cua da thuc

Sau khi che bonnet, bird view chi con du lieu o phia tren. Da thuc bac hai thi du tinh ca phan duoi, va duong cong bao cao cho xuat toan bo khung hinh nen phan gan xe la suy luong. Khi thay bang hinh anh, can kiem tra duong cong gan day co buoc vang khong; neu van, day la ly do bo hanh che bonnet va ghi ro trong bao cao thu.

## 11.3. Nguong mau can hieu chinh theo anh that

Hien tai LANE_MIN_VALUE = 0, LANE_MAX_VALUE = 110 cho san gach kem ~225. Neu den tran lam vach sat trang, nen se khong con nam trong dai 0..110 va mask rong. Cach dung: bat ENABLE_LANE_COLOR_SAMPLE va in ra lane_color_bgr de do gia tri that cua vach roi chinh dai nguong.

## 11.4. Diem da dung, nen giu nguyen

Khong dung Hough: vach trong nhin lau co goc nho, Hough de lech va ton tinh.

Camera doc lap - nhanh xu ly rieng cho tach dong cat hinh, latest-frame de khong cong don tre.

Loi an toan mac dinh: valid = false thi camera_cmd = STOP, khong bao gio giu lenh cu.

Khong dung delay trong vong xu ly.

# 12. Ket luan va bang kiem chung

CameraLane hien thuc phuong phap cua master: AND hai nguong (Sobel X tren kenh G va nguong kenh R) de tach vach, IPM 320x240, 9 cua so truot, fit bac hai tren pixel, chieu nguoc ve anh goc. Thay doi duy nhat so voi master la doi chieu nguong mau cho vach cao su den (LANE_MIN_VALUE/MAX_VALUE) va CLAHE tren kenh G de bu sang khong deu. Co them nhanh du phong Canny + quet dong va cac dieu kien tu choi de khong bao gio steer bang duong cong sai.

## 12.1. Ket qua kiem chung tren 14 mo hinh tong hop

Moc hinh bo phong doi chieu cau sai truoc day (lan hep o duoi khung hinh, tuc goc tu nam DUOI khung - sai vat ly camera nhin xuong). Sau khi sua lai goc tu nam TREN khung va khoi phuc dung pipeline master, ket qua nhu sau:

| Tinh huong | Nhanh | dev (px) | Ket qua |
| --- | --- | --- | --- |
| Duong thang, vach den tren san kem | IPM | -1 | FWD, hai lan |
| Lan lech trai | Du phong | -70 | LEFT |
| Lan lech phai | Du phong | +80 | RIGHT |
| Cua, dinh cong sang trai | Du phong | -3 | FWD |
| Duong ch e + vach | IPM | +1 | FWD, bo qua duong ch e |
| Tuong phan thap 168/192 | IPM | -7 | FWD, hai lan |
| Tuong phan thap 150/180 | IPM | -1 | FWD, hai lan |
| Vet sang den tran | IPM | -1 | FWD, hai lan |
| Nhieu muoi bi tren san | IPM | -6 | FWD, hai lan |
| Lan rong (gap 330) | IPM | 0 | FWD, hai lan |
| Lan hep (gap 150) | Du phong | 0 | FWD |
| Chi co mot vach | - | - | Khong ra lenh |
| Khong co vach | - | - | Khong ra lenh |
| Cua o toc do 8 km/h | Du phong | -4 | FWD |

## 12.2. Danh sach kiem tra khi chay that tren xe

Chay may khong tach: khong lane, khong duong ch e, khong vat sang mau den.

Dat xe giua hai vach, di thang: dev phai nho (|dev| < 15 px) va lenh FWD.

Tien goc phai 10-15 cm: dev phai am (LEFT), lan phai la lan gan xe.

Che dap den tran tren san: he thong phai chuyen du phong, khong bao gio STOP giat.

Do lai mau vach that (lane_color_bgr) va hieu chinh LANE_MIN_VALUE/MAX_VALUE.

Do va nhin hinh thang tren mot frame that de hieu chinh 4 diem IPM va vung bonnet.

Kiem tra an toan: mat cam, khong vach, vach che nua, vach lech ngoai khung.

Ket luan: don mot lan: tach vach bang AND hai nguong, IPM bird's-eye, 9 cua so truot, fit bac hai tren pixel, chieu nguoc ve anh goc.